Beach-Side RC Oil Cleaning Machine: An Embedded Systems Approach for

Coastal Oil Spill Recovery

System Manual

Submitted by:

Dela Cruz, Denver M.

Delin, Princess Angel G.

Merano, John Lloyd S.

Sombilon, Lyndhel M.

Alejandro, Jhonzen Carl C.

Escuterio, Kazz Denielle

Magararu, Joanne

Malabha, Jay-ar

Magno, Carlo Jose

May, 2026

Table Of Contents

Introduction	3

System Overview	3

Main Components	4

Safety Guidelines	5

Power System Overview	5

Initial Setup Procedure	6

Connecting the Machine to Wi-Fi	6

Identifying the IP Address	7

Accessing the Web Controller	8

Web Controller Interface Guide	8

Operating the Machine	10

Emergency Stop Procedure	11

Charging System and Solar Setup	11

Maintenance Procedures	11

Troubleshooting and Debugging Guide	12

Wiring and Electronics Notes	15

Recommended Startup and Shutdown Sequence	16

Storage Guidelines	16

Technical Specifications	17

Developer Notes	17

# Introduction

The Beach-Side RC Oil Cleaning Machine is a remote-controlled oil skimming and collection machine designed for cleaning oil-contaminated water surfaces near shorelines, beaches, and shallow waters. The machine uses a web-based control system powered by an ESP32 microcontroller and can be operated through any smartphone, tablet, or laptop connected to the same Wi-Fi network.

The machine includes:

- Dual mobility motors for movement

- Drum skimmer system for oil collection

- Diaphragm pump for oil transfer

- Linear actuator for boom arm positioning

- Web-based remote-control interface

- Solar charging support

- Battery-powered operation

# System Overview

The system consists of four major sections:

A. Mobility System

Responsible for machine movement using two 12V MY6812 120W brush motors controlled by BTS7960 motor drivers.

B. Oil Collection System

Consists of:

- Drum skimmer motor

- Drum skimmer assembly

- Diaphragm pump

- Oil collection container

C. Boom Arm and Actuator System

Uses a linear actuator to raise or lower the boom/skimmer assembly.

D. Control and Electronics System

Consists of:

- ESP32 microcontroller

- WiFi web controller

- MOSFET modules

- BTS7960 motor drivers

- Fuse block

- Buck converter

- Battery and solar charging system

# Main Components

Mechanical Components

- Steel or aluminum chassis frame

- Drum skimmer assembly

- Boom arm assembly

- Linear actuator

- Front swivel caster wheel

- Rear drive wheels

- Chain and sprocket system

Electrical Components

- ESP32 microcontroller

- 12V 25Ah battery

- 50W solar panel

- 30A solar charge controller (SCC)

- 50A 2-pole circuit breaker

- 10A 2-pole circuit breaker

- 12-way marine fuse block

- BTS7960 motor drivers

- MOSFET modules

- 5A buck converter

Mobility Motors

- MY6812 12V 120W brush motors

Auxiliary Devices

- 12V diaphragm pump

- 12V JGB37 drum motor

# Safety Guidelines

General Safety

- Never operate the machine without checking all wiring connections.

- Keep hands away from moving chains, sprockets, and drum assemblies.

- Do not expose electronics directly to seawater.

- Always inspect wires for damage before operation.

- Disconnect battery power before maintenance.

Electrical Safety

- Ensure proper fuse ratings are installed.

- Never short positive and negative terminals.

- Keep the battery dry and ventilated.

- Use only properly rated wires.

Water Operation Safety

- Do not operate in deep water beyond machine capability.

- Avoid strong waves or currents.

- Ensure flotation stability before operation.

# Power System Overview

Solar Charging Connection

The charging system is connected as follows:

50W Solar Panel → 10A Circuit Breaker → 30A SCC → 50A Circuit Breaker → 12V Battery

Main Power Distribution

The battery connects directly to:

- Marine fuse block positive terminal

- Marine fuse block negative terminal

- 50A breaker

All system components receive power through the marine fuse block.

Fuse Ratings

| Component | Fuse Rating |
| --- | --- |
| Left Mobility Motor | 20A |
| Right Mobility Motor | 20A |
| Linear Actuator | 20A |
| Diaphragm Pump | 15A |
| Drum Motor | 5A |
| Buck Converter | 5A |

# Initial Setup Procedure

Step 1: Inspect the Machine

Before powering on:

- Check all wiring connections

- Check battery voltage

- Ensure fuses are installed

- Ensure chains and sprockets are aligned

- Verify wheels rotate properly

Step 2: Turn ON the Main Breaker

Switch ON:

- 50A battery breaker

- 10A solar breaker if charging is needed

Step 3: Power the ESP32

The ESP32 receives 5V power from the buck converter.

After power-up:

- ESP32 will automatically initialize

- WiFi network will activate

- Web server will start

# Connecting the Machine to Wi-Fi

The ESP32 creates or connects to a Wi-Fi network depending on the programmed configuration.

Option A: ESP32 Access Point Mode

If configured as Access Point:

- Open phone Wi-Fi settings

- Look for the machine Wi-Fi name (SSID)

- Connect using the configured password

- Wait until connected

Example:

- Wi-Fi Name: Galaxy A52s 5GB1D0

- Password: upui9397

Option B: Home Router Mode

If connected to a router:

- Connect phone to same WiFi network

- Power on machine

- Find machine IP address

- Open browser and enter IP address

Note: The Wi-Fi SSID should be modified to “Galaxy A52s 5GB1D0” and Password to “upui9397”.

# Identifying the IP Address

Method 1: Serial Monitor (Recommended)

Connect ESP32 to a laptop.

Open Arduino Serial Monitor.

The ESP32 will display:

IP Address: 192.168.1.100

Method 2: Router Device List

Open router admin page.

Check connected devices.

Look for:

- ESP32

- OilCleaner

- Unknown device

Example IP:

- 192.168.1.100

# Accessing the Web Controller

Procedure

- Open any browser

- Type the IP address into the address bar

- Press Enter

Example:

http://192.168.1.100

The controller dashboard will load.

# Web Controller Interface Guide

The web controller interface allows complete remote operation of the machine.

A. Mobility Control Joystick

The large circular joystick controls movement.

Functions

| Direction | Action |
| --- | --- |
| Forward | Move forward |
| Backward | Reverse |
| Left | Turn left |
| Right | Turn right |
| Center | Stop |

B. Motor Speed Indicators

The interface displays:

- Left motor speed percentage

- Right motor speed percentage

Example:

- LEFT: 60%

- RIGHT: 60%

C. Pump Control

Controls the diaphragm pump.

Button Functions

| Button | Function |
| --- | --- |
| Power Button ON | Start pump |
| Power Button OFF | Stop pump |

D. Auxiliary Motor Control

Controls the drum skimmer motor.

Button Functions

| Button | Function |
| --- | --- |
| Power Button ON | Start drum motor |
| Power Button OFF | Stop drum motor |

E. Linear Actuator Controls

Controls boom arm movement.

| Button | Function |
| --- | --- |
| Up Arrow | Extend actuator |
| Down Arrow | Retract actuator |

F. Emergency Stop

The red EMERGENCY STOP button immediately disables:

- Mobility motors

- Pump

- Drum motor

- Linear actuator

Use during unsafe operation.

# Operating the Machine

Starting Operation

- Power ON system

- Connect to web controller

- Test motor movement slowly

- Lower boom arm using actuator

- Turn ON drum motor

- Turn ON pump

- Begin forward movement

Oil Collection Process

- Drum rotates and attracts oil

- Oil is scraped into collection tray

- Pump transfers oil into storage container

Steering Tips

- Use slow movements in shallow water

- Avoid sudden turns

- Reduce speed during skimming

# Emergency Stop Procedure

If unsafe behavior occurs:

- Press EMERGENCY STOP button immediately

- Turn OFF main breaker if needed

- Disconnect battery during maintenance

Possible emergency situations:

- Motor overheating

- Wire smoke

- Water entering electronics

- Mechanical jam

- Loss of control

# Charging System and Solar Setup

Solar Charging Procedure

- Place solar panel under sunlight

- Turn ON 10A solar breaker

- SCC will regulate charging automatically

SCC Indicators

| Indicator | Meaning |
| --- | --- |
| Charging Icon | Solar charging active |
| Battery Icon | Battery connected |
| Low Voltage | Battery low |

Important Notes

- Never connect SCC directly to panel before battery

- Always connect battery first

- Use proper wire sizes

# Maintenance Procedures

Daily Inspection

Check:

- Battery voltage

- Fuse condition

- Motor temperature

- Wire connections

- Chain tension

- Pump hoses

Weekly Maintenance

- Clean drum skimmer

- Remove oil residue

- Inspect sprockets

- Tighten bolts

- Inspect actuator mounting

Monthly Maintenance

- Check bearings

- Reapply grease if needed

- Inspect waterproof seals

- Test all breakers

# Troubleshooting and Debugging Guide

A. ESP32 Does Not Power ON

Possible Causes

- Buck converter failure

- No battery power

- Loose wiring

- Fuse blown

Solutions

- Check 5V output from buck converter

- Check battery voltage

- Replace fuse

- Verify GND and 5V wiring

B. Cannot Access Web Controller

Possible Causes

- Wrong IP address

- WiFi not connected

- ESP32 frozen

Solutions

- Restart ESP32

- Reconnect to WiFi

- Check Serial Monitor IP address

- Ensure same network connection

C. Mobility Motors Not Moving

Possible Causes

- BTS7960 wiring issue

- Blown fuse

- Low battery

- Loose motor wires

Solutions

- Check BTS7960 B+ and B-

- Check RPWM and LPWM GPIO connections

- Verify fuse continuity

- Test battery voltage

D. Pump Not Working

Possible Causes

- MOSFET wiring issue

- Pump fuse blown

- Pump jammed

Solutions

- Check MOSFET IN+ and IN-

- Check GPIO control signal

- Replace fuse

- Test pump directly with battery

E. Drum Motor Not Spinning

Possible Causes

- MOSFET issue

- Motor jam

- Broken shaft coupling

Solutions

- Check MOSFET output voltage

- Inspect drum shaft

- Test motor independently

F. Linear Actuator Not Moving

Possible Causes

- BTS7960 issue

- Mechanical obstruction

- Actuator overload

Solutions

- Check actuator fuse

- Check BTS7960 enable pins

- Inspect actuator mounting

- Remove obstruction

G. System Randomly Restarts

Possible Causes

- Voltage drop

- Weak battery

- Shared power noise

Solutions

- Add capacitor near ESP32

- Check battery condition

- Separate high-current wiring from signal wires

H. Motors Move in Wrong Direction

Solution

Swap motor terminal polarity:

- Reverse M+ and M-

I. Overheating Wires

Possible Causes

- Wire too small

- Loose terminal

- Excess current

Solutions

- Use thicker wire

- Tighten connections

- Reduce load

# Wiring and Electronics Notes

Important Wiring Rules

- Keep signal wires away from motor wires

- Use proper fuse sizes

- Secure all terminals tightly

- Waterproof exposed connections

Wire Recommendations

| Application | Recommended Wire |
| --- | --- |
| Battery Main Line | 8-12AWG |
| Mobility Motors | 16AWG |
| Pump | 18AWG |
| Drum Motor | 18AWG |
| ESP32 | 20–22AWG |
| Signal Wires | 22AWG |

# Recommended Startup and Shutdown Sequence

Startup

- Check machine

- Turn ON breakers

- Wait for ESP32 boot

- Connect to controller

- Test controls

- Begin operation

Shutdown

- Stop movement

- Turn OFF pump

- Turn OFF drum motor

- Retract actuator

- Turn OFF breakers

- Disconnect battery if storing

# Storage Guidelines

- Store in dry location

- Disconnect battery during long storage

- Recharge battery monthly

- Cover electronics from moisture

- Clean oil residues after use

# Technical Specifications

| Component | Specification |
| --- | --- |
| Battery | 12V 25Ah Gel Battery |
| Solar Panel | 50W |
| SCC | 30A PWM/MPPT |
| Mobility Motors | MY6812 12V 120W |
| Drum Motor | JGB37 12V 35RPM |
| Pump | 12V Diaphragm Pump |
| Controller | ESP32 |
| Main Breaker | 50A |
| Solar Breaker | 10A |

# Developer Notes

GPIO Assignment

Mobility Driver #1

| Function | GPIO |
| --- | --- |
| RPWM1 | 25 |
| LPWM1 | 26 |
| R_EN1 | 27 |
| L_EN1 | 13 |

Mobility Driver #2

| Function | GPIO |
| --- | --- |
| RPWM2 | 32 |
| LPWM2 | 33 |
| R_EN2 | 18 |
| L_EN2 | 19 |

Linear Actuator Driver

| Function | GPIO |
| --- | --- |
| RPWM3 | 23 |
| LPWM3 | 22 |
| R_EN3 | 21 |
| L_EN3 | 4 |

MOSFET Outputs

| Device | GPIO |
| --- | --- |
| Pump | 14 |
| Drum Motor | 12 |